Supplementary Material

S1. Available data, software, and computational environment

This supplement describes the data, computational environment, and analyses that support the reported results. It includes the archived forward-model diagnostic, scale-invariant kernel selection, nested independent-test results, ten-run optimizer comparisons, surrogate-conditioned search dispersion, in-domain forward checks, representative candidates, and reproduction of the grey relational analysis.

| Item | Setting or available information |
| --- | --- |
| PFC source version | PFC 5.0 |
| MATLAB version | R2024b, 24.2, 64-bit |
| Operating system | Windows 11 Pro, 64-bit |
| Hardware | AMD Ryzen AI 9 HX 370; 12 cores/24 logical processors; 31.1 GB RAM |
| Saved GPR models | The production bundle was generated in MATLAB R2024b. Peak stress and Poisson’s ratio use ARD Matern 3/2, whereas peak strain and elastic modulus use ARD Matern 5/2. The bundle is supplied as round2_production_models.mat. |
| Neural-network comparator | MATLAB fitrnet; one hidden layer with 10 nodes |
| Preprocessing | Training-derived min-max scaling to [0,1] |
| Controlled optimizers | Standard NSGA-II and NSGA-III; population 60; 30 generations; ten independent runs per target; 12 targets; matched initialization, variation, bounds, and repair. |
| Diversity mechanism | NSGA-II uses crowding distance and no reference points; NSGA-III uses ten reference-point divisions (286 simplex points). |
| PFC2D calls during optimizer search | 0 for NSGA-II and NSGA-III |
| Boundary handling | Every offspring from both algorithms is repaired to the five reviewed training-domain bounds after variation. |

The modelling workbook contains 3125 observations. No duplicate full rows or duplicate combinations of the first five mesoscopic-parameter columns were identified. Each of the five parameters occurs at exactly five levels; together, the 3125 unique combinations constitute the complete 55 full-factorial design.

S2. Repeated disjoint-split evaluation of the forward surrogates

Table S1 contains the archived round-1 diagnostic. In each of five deterministic disjoint 70/10/20 splits, a single common kernel was chosen by the mean of four response-normalized validation errors and then refitted on training plus validation data. It is distinct from the later per-response production selection in Section S6. The restored model_diagnostics directory contains its scripts, selection records and run-level metrics.

Table S1. Archived repeated disjoint-split forward-model diagnostic

| Response | Model | n | RMSE | MARE | R² |
| --- | --- | --- | --- | --- | --- |
| Elastic modulus | GPR | 5 | 1.4356 ± 0.0546 | 0.0162 ± 0.0004 | 0.9868 ± 0.0011 |
| Elastic modulus | NN | 5 | 2.8352 ± 0.0910 | 0.0712 ± 0.0005 | 0.9484 ± 0.0039 |
| Peak strain | GPR | 5 | 0.0810 ± 0.0019 | 0.0416 ± 0.0017 | 0.9887 ± 0.0003 |
| Peak strain | NN | 5 | 0.1072 ± 0.0057 | 0.0641 ± 0.0036 | 0.9802 ± 0.0020 |
| Peak stress | GPR | 5 | 1.5554 ± 0.0712 | 0.0276 ± 0.0004 | 0.9879 ± 0.0006 |
| Peak stress | NN | 5 | 5.7190 ± 0.8412 | 0.1150 ± 0.0199 | 0.8318 ± 0.0487 |
| Poisson's ratio | GPR | 5 | 0.0067 ± 0.0002 | 0.0144 ± 0.0009 | 0.9935 ± 0.0005 |
| Poisson's ratio | NN | 5 | 0.0102 ± 0.0013 | 0.0264 ± 0.0039 | 0.9844 ± 0.0043 |

Values are arithmetic mean ± sample SD across five repeats, with metrics calculated on the held-out cases of each repeat. Training, validation and test sets are disjoint within a repeat; observations may recur across repeats. Table S1 is an archived diagnostic, not the current production-model evaluation.

S3. Repeated direct inverse-model benchmarks

Table S2. Repeated evaluation of the direct inverse-model baselines

| Model | Mesoscopic parameter | n | RMSE | MARE | R² |
| --- | --- | --- | --- | --- | --- |
| GPR | Cohesion | 10 | 4.5125 ± 0.1549 | 0.1531 ± 0.0118 | 0.5967 ± 0.0251 |
| GPR | Bond effective modulus | 10 | 2.6327 ± 0.3514 | 0.0179 ± 0.0017 | 0.9842 ± 0.0041 |
| GPR | Friction angle | 10 | 19.0225 ± 0.3970 | 0.5566 ± 0.0268 | 0.1993 ± 0.0218 |
| GPR | Bond normal-to-shear stiffness ratio | 10 | 0.0771 ± 0.0066 | 0.0115 ± 0.0011 | 0.9878 ± 0.0020 |
| GPR | Tensile strength | 10 | 19.5601 ± 0.6914 | 0.3861 ± 0.0459 | 0.5210 ± 0.0392 |
| NN | Cohesion | 10 | 5.1662 ± 0.1782 | 0.1794 ± 0.0085 | 0.4714 ± 0.0327 |
| NN | Bond effective modulus | 10 | 3.3724 ± 0.3206 | 0.0329 ± 0.0035 | 0.9743 ± 0.0052 |
| NN | Friction angle | 10 | 19.4958 ± 0.3543 | 0.5678 ± 0.0225 | 0.1589 ± 0.0195 |
| NN | Bond normal-to-shear stiffness ratio | 10 | 0.1100 ± 0.0029 | 0.0357 ± 0.0025 | 0.9753 ± 0.0016 |
| NN | Tensile strength | 10 | 21.7578 ± 1.2331 | 0.4849 ± 0.0459 | 0.4072 ± 0.0609 |

Table 6 and Fig. 6 summarise the direct inverse-modelling analysis reported in Supplementary Table S2. The archived analysis used ten repeated 80/20 splits, with disjoint training and test cases within each repeat; cases may recur across repeats. The inverse GPR used an ARD rational-quadratic kernel, and the NN comparator was a 10-node MATLAB fitrnet model. These diagnostic models are distinct from the four forward GPR models used in optimisation and from the earlier BP illustration. Metrics were calculated on the held-out subset of each repeat and are reported as the arithmetic mean ± sample SD across the ten repeats, without pooling predictions. MSE was obtained by squaring each run’s RMSE before aggregation, rather than by squaring the mean RMSE. Mean test R² was 0.5210 for tensile strength and 0.1993 for friction angle. These results are consistent with weak inverse constraints, but do not by themselves quantify physical identifiability.

S4. Repeated training-set-size sensitivity analysis

Table S3. Repeated learning-curve results under a common test design

| Training cases | Common test cases | Response | n | RMSE | MARE | R² |
| --- | --- | --- | --- | --- | --- | --- |
| 400 | 625 | Elastic modulus | 5 | 1.6538 ± 0.2122 | 0.0208 ± 0.0014 | 0.9822 ± 0.0051 |
| 400 | 625 | Peak strain | 5 | 0.0995 ± 0.0063 | 0.0537 ± 0.0027 | 0.9830 ± 0.0020 |
| 400 | 625 | Peak stress | 5 | 1.8833 ± 0.1241 | 0.0332 ± 0.0010 | 0.9823 ± 0.0016 |
| 400 | 625 | Poisson's ratio | 5 | 0.0077 ± 0.0002 | 0.0181 ± 0.0017 | 0.9914 ± 0.0003 |
| 800 | 625 | Elastic modulus | 5 | 1.6016 ± 0.2187 | 0.0194 ± 0.0006 | 0.9833 ± 0.0049 |
| 800 | 625 | Peak strain | 5 | 0.0894 ± 0.0034 | 0.0469 ± 0.0009 | 0.9863 ± 0.0008 |
| 800 | 625 | Peak stress | 5 | 1.7412 ± 0.0940 | 0.0308 ± 0.0005 | 0.9848 ± 0.0011 |
| 800 | 625 | Poisson's ratio | 5 | 0.0074 ± 0.0001 | 0.0167 ± 0.0009 | 0.9920 ± 0.0004 |
| 1200 | 625 | Elastic modulus | 5 | 1.4800 ± 0.0651 | 0.0182 ± 0.0007 | 0.9860 ± 0.0012 |
| 1200 | 625 | Peak strain | 5 | 0.0858 ± 0.0038 | 0.0445 ± 0.0021 | 0.9873 ± 0.0010 |
| 1200 | 625 | Peak stress | 5 | 1.6873 ± 0.0870 | 0.0296 ± 0.0004 | 0.9858 ± 0.0010 |
| 1200 | 625 | Poisson's ratio | 5 | 0.0072 ± 0.0001 | 0.0161 ± 0.0010 | 0.9924 ± 0.0003 |
| 1600 | 625 | Elastic modulus | 5 | 1.4475 ± 0.0519 | 0.0174 ± 0.0004 | 0.9866 ± 0.0010 |
| 1600 | 625 | Peak strain | 5 | 0.0840 ± 0.0031 | 0.0435 ± 0.0020 | 0.9879 ± 0.0009 |
| 1600 | 625 | Peak stress | 5 | 1.6178 ± 0.0765 | 0.0287 ± 0.0006 | 0.9869 ± 0.0007 |
| 1600 | 625 | Poisson's ratio | 5 | 0.0071 ± 0.0001 | 0.0158 ± 0.0009 | 0.9926 ± 0.0004 |
| 2000 | 625 | Elastic modulus | 5 | 1.4272 ± 0.0601 | 0.0169 ± 0.0002 | 0.9869 ± 0.0011 |
| 2000 | 625 | Peak strain | 5 | 0.0825 ± 0.0033 | 0.0425 ± 0.0022 | 0.9883 ± 0.0009 |
| 2000 | 625 | Peak stress | 5 | 1.5905 ± 0.0802 | 0.0283 ± 0.0006 | 0.9874 ± 0.0008 |
| 2000 | 625 | Poisson's ratio | 5 | 0.0070 ± 0.0002 | 0.0154 ± 0.0008 | 0.9928 ± 0.0006 |
| 2500 | 625 | Elastic modulus | 5 | 1.4291 ± 0.0518 | 0.0167 ± 0.0004 | 0.9869 ± 0.0010 |
| 2500 | 625 | Peak strain | 5 | 0.0815 ± 0.0026 | 0.0419 ± 0.0019 | 0.9886 ± 0.0006 |
| 2500 | 625 | Peak stress | 5 | 1.5581 ± 0.0707 | 0.0278 ± 0.0004 | 0.9879 ± 0.0006 |
| 2500 | 625 | Poisson's ratio | 5 | 0.0069 ± 0.0001 | 0.0148 ± 0.0009 | 0.9931 ± 0.0004 |

Fig. 7 presents the fixed-test learning curves reported in Supplementary Table S3. This archived analysis used a fixed ARD rational-quadratic GPR kernel and does not evaluate the later response-specific production models. Within each of five repeats, the same 625-case held-out subset was used for nested training subsets of 400, 800, 1200, 1600, 2000 and 2500 cases. Scaling was estimated from the training subset at each size. Points and error bars show the mean and sample SD of held-out RMSE across the five repeats. Errors generally decreased as the training set grew, although the gains at larger sizes were smaller and varied among responses. These curves describe sample-size effects within the factorial design domain; they do not identify a universal convergence threshold. The response-specific production models were subsequently fitted to all 3125 cases. This full-data fit is not an additional held-out point on the learning curve.

S5. Controlled surrogate-domain optimizer benchmarks

Table S4. Aggregate ten-run NSGA-II and NSGA-III comparison

| Algorithm | Target-run records | Best error | IGD | Hypervolume | Runtime / s | Evaluations / run | PFC2D calls |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NSGA-II | 120 | 0.02708 ± 0.02908 | 0.06071 ± 0.05473 | 0.989617 ± 0.029486 | 1.849 ± 0.167 | 1860 | 0 |
| NSGA-III | 120 | 0.00602 ± 0.00476 | 0.03782 ± 0.01944 | 0.999329 ± 0.000811 | 1.896 ± 0.128 | 1860 | 0 |

Table S5. Target-level ten-run NSGA-II and NSGA-III comparison (mean ± SD)

| Target | Algorithm | n | Best error | IGD | Hypervolume | Runtime / s | Evaluations / run | PFC2D calls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | NSGA-II | 10 | 0.01296 ± 0.00501 | 0.03535 ± 0.00911 | 0.998883 ± 0.000955 | 1.838 ± 0.124 | 1860 | 0 |
| 1 | NSGA-III | 10 | 0.00205 ± 0.00116 | 0.02631 ± 0.00735 | 0.999819 ± 0.000151 | 1.865 ± 0.145 | 1860 | 0 |
| 2 | NSGA-II | 10 | 0.01233 ± 0.00571 | 0.03890 ± 0.01406 | 0.999322 ± 0.000558 | 1.796 ± 0.087 | 1860 | 0 |
| 2 | NSGA-III | 10 | 0.00744 ± 0.00517 | 0.03304 ± 0.00684 | 0.999209 ± 0.000766 | 2.011 ± 0.230 | 1860 | 0 |
| 3 | NSGA-II | 10 | 0.01162 ± 0.00756 | 0.03393 ± 0.01559 | 0.998762 ± 0.001430 | 1.989 ± 0.085 | 1860 | 0 |
| 3 | NSGA-III | 10 | 0.00137 ± 0.00077 | 0.02183 ± 0.00155 | 0.999890 ± 0.000055 | 1.934 ± 0.058 | 1860 | 0 |
| 4 | NSGA-II | 10 | 0.02127 ± 0.01027 | 0.05217 ± 0.02062 | 0.997060 ± 0.003042 | 1.999 ± 0.167 | 1860 | 0 |
| 4 | NSGA-III | 10 | 0.00309 ± 0.00165 | 0.04218 ± 0.00585 | 0.999747 ± 0.000168 | 1.852 ± 0.072 | 1860 | 0 |
| 5 | NSGA-II | 10 | 0.01422 ± 0.01226 | 0.03652 ± 0.02375 | 0.998471 ± 0.002088 | 1.962 ± 0.356 | 1860 | 0 |
| 5 | NSGA-III | 10 | 0.00273 ± 0.00249 | 0.02414 ± 0.00294 | 0.999777 ± 0.000228 | 1.768 ± 0.028 | 1860 | 0 |
| 6 | NSGA-II | 10 | 0.02316 ± 0.01007 | 0.04411 ± 0.01807 | 0.996859 ± 0.002964 | 1.842 ± 0.149 | 1860 | 0 |
| 6 | NSGA-III | 10 | 0.01141 ± 0.00157 | 0.03372 ± 0.00187 | 0.998214 ± 0.001254 | 1.926 ± 0.100 | 1860 | 0 |
| 7 | NSGA-II | 10 | 0.02022 ± 0.00857 | 0.04559 ± 0.01625 | 0.998366 ± 0.001191 | 1.787 ± 0.122 | 1860 | 0 |
| 7 | NSGA-III | 10 | 0.00408 ± 0.00345 | 0.01929 ± 0.00377 | 0.999657 ± 0.000374 | 2.048 ± 0.108 | 1860 | 0 |
| 8 | NSGA-II | 10 | 0.01544 ± 0.00805 | 0.03697 ± 0.01637 | 0.998881 ± 0.001053 | 1.946 ± 0.136 | 1860 | 0 |
| 8 | NSGA-III | 10 | 0.00588 ± 0.00408 | 0.02379 ± 0.00503 | 0.999576 ± 0.000308 | 1.901 ± 0.049 | 1860 | 0 |
| 9 | NSGA-II | 10 | 0.03145 ± 0.00848 | 0.07005 ± 0.00960 | 0.994547 ± 0.003730 | 1.798 ± 0.030 | 1860 | 0 |
| 9 | NSGA-III | 10 | 0.01322 ± 0.00450 | 0.07502 ± 0.00656 | 0.998420 ± 0.001163 | 1.921 ± 0.073 | 1860 | 0 |
| 10 | NSGA-II | 10 | 0.03607 ± 0.02412 | 0.07842 ± 0.05205 | 0.990267 ± 0.010639 | 1.746 ± 0.049 | 1860 | 0 |
| 10 | NSGA-III | 10 | 0.00515 ± 0.00338 | 0.04218 ± 0.01180 | 0.999549 ± 0.000367 | 1.897 ± 0.119 | 1860 | 0 |
| 11 | NSGA-II | 10 | 0.01878 ± 0.00612 | 0.04543 ± 0.01120 | 0.997390 ± 0.001545 | 1.762 ± 0.058 | 1860 | 0 |
| 11 | NSGA-III | 10 | 0.00578 ± 0.00247 | 0.03865 ± 0.00363 | 0.999339 ± 0.000431 | 1.831 ± 0.074 | 1860 | 0 |
| 12 | NSGA-II | 10 | 0.10746 ± 0.03668 | 0.21103 ± 0.06738 | 0.906597 ± 0.053901 | 1.722 ± 0.029 | 1860 | 0 |
| 12 | NSGA-III | 10 | 0.01008 ± 0.00378 | 0.07365 ± 0.01793 | 0.998755 ± 0.000750 | 1.793 ± 0.054 | 1860 | 0 |

NSGA-II and NSGA-III minimized the four target-normalized response errors as separate objectives. NSGA-II maintained diversity by crowding distance, whereas NSGA-III used reference-point environmental selection. Both algorithms used a population of 60 for 30 generations, with the same initialization procedure, variation settings, and post-variation boundary repair.

IGD uses the pooled nondominated union of both algorithms and all runs for each target as the empirical reference set. Hypervolume is estimated in clipped [0,1]4 objective space using reference point [1,1,1,1] and 100000 fixed-seed Monte Carlo samples. The two algorithms use distinct recorded seeds; results are therefore reported as independent-run mean ± SD rather than paired tests.

Across the 12 target-level means, NSGA-III yielded a lower best error for all targets, a lower IGD for 11 targets, and a higher hypervolume for 11 targets. Target 9 was the IGD exception, and target 2 was the hypervolume exception. These results support a limited conclusion about reference-point selection under the present four-objective budget, rather than general superiority.

Both optimizers searched the saved production GPR surrogates without calling PFC2D. The reported runtimes exclude generation of the 3125-case PFC2D dataset and direct PFC2D verification. The scalar-GA benchmark was not included in this comparison.

Fig. S1. Optimiser comparison and surrogate-conditioned parameter dispersion.

(a, b) Target-1 run-1 objective projections; (c) best mean relative error; (d) IGD; (e) hypervolume gap (1 − HV).

Points and error bars in (c–e) are mean ± sample SD across ten independent runs per target.

(f) Mean normalized parameter IQR, calculated within each target after a common target-best + 0.005 filter and exact within-target deduplication,

then averaged equally over 12 targets. These bars describe search dispersion, not physical uncertainty.

S6. Scale-invariant, per-response kernel selection

For response j in split r, the selection score is validation RMSE divided by the range of response j in that split’s training subset. Scores are dimensionless and are never added across MPa, ‰, GPa, and Poisson-ratio units. Table S6 gives all 40 five-split mean ± SD values from the completed MATLAB R2024b refit; split-level values, seeds, training minima/maxima, and rankings are supplied as CSV.

Table S6. Per-response validation NRMSE for all candidate kernels

| Response | Rank | Kernel | Mean validation NRMSE | SD |
| --- | --- | --- | --- | --- |
| Peak stress | 1 | ARD Matern 3/2 | 0.0221287 | 0.0006283 |
| Peak stress | 2 | ARD Rational Quadratic | 0.0223445 | 0.0007452 |
| Peak stress | 3 | ARD Matern 5/2 | 0.0223523 | 0.0007389 |
| Peak stress | 4 | ARD Exponential | 0.0226273 | 0.0009732 |
| Peak stress | 5 | ARD Squared Exponential | 0.0227502 | 0.0008897 |
| Peak stress | 6 | Rational Quadratic | 0.0246862 | 0.0008278 |
| Peak stress | 7 | Matern 5/2 | 0.0247490 | 0.0008459 |
| Peak stress | 8 | Squared Exponential | 0.0249272 | 0.0009317 |
| Peak stress | 9 | Matern 3/2 | 0.0251704 | 0.0008087 |
| Peak stress | 10 | Exponential | 0.0284836 | 0.0006842 |
| Peak strain | 1 | ARD Matern 5/2 | 0.0153950 | 0.0008517 |
| Peak strain | 2 | ARD Rational Quadratic | 0.0155130 | 0.0008292 |
| Peak strain | 3 | ARD Matern 3/2 | 0.0155297 | 0.0006623 |
| Peak strain | 4 | ARD Squared Exponential | 0.0157143 | 0.0007973 |
| Peak strain | 5 | Rational Quadratic | 0.0162815 | 0.0009285 |
| Peak strain | 6 | Matern 5/2 | 0.0162829 | 0.0007928 |
| Peak strain | 7 | ARD Exponential | 0.0163952 | 0.0006926 |
| Peak strain | 8 | Squared Exponential | 0.0164233 | 0.0009718 |
| Peak strain | 9 | Matern 3/2 | 0.0166321 | 0.0009652 |
| Peak strain | 10 | Exponential | 0.0216569 | 0.0020739 |
| Elastic modulus | 1 | ARD Matern 5/2 | 0.0274769 | 0.0043282 |
| Elastic modulus | 2 | ARD Rational Quadratic | 0.0275250 | 0.0042723 |
| Elastic modulus | 3 | Rational Quadratic | 0.0275712 | 0.0044407 |
| Elastic modulus | 4 | Squared Exponential | 0.0276358 | 0.0043753 |
| Elastic modulus | 5 | ARD Squared Exponential | 0.0276362 | 0.0042324 |
| Elastic modulus | 6 | Matern 5/2 | 0.0276732 | 0.0045338 |
| Elastic modulus | 7 | ARD Matern 3/2 | 0.0277809 | 0.0045683 |
| Elastic modulus | 8 | Matern 3/2 | 0.0279049 | 0.0049563 |
| Elastic modulus | 9 | ARD Exponential | 0.0298504 | 0.0039408 |
| Elastic modulus | 10 | Exponential | 0.0301741 | 0.0051480 |
| Poisson’s ratio | 1 | ARD Matern 3/2 | 0.0204143 | 0.0023882 |
| Poisson’s ratio | 2 | ARD Matern 5/2 | 0.0207229 | 0.0022040 |
| Poisson’s ratio | 3 | ARD Rational Quadratic | 0.0210141 | 0.0022685 |
| Poisson’s ratio | 4 | Matern 3/2 | 0.0212282 | 0.0020080 |
| Poisson’s ratio | 5 | Matern 5/2 | 0.0212467 | 0.0020333 |
| Poisson’s ratio | 6 | ARD Exponential | 0.0213785 | 0.0021051 |
| Poisson’s ratio | 7 | ARD Squared Exponential | 0.0214285 | 0.0021411 |
| Poisson’s ratio | 8 | Rational Quadratic | 0.0214450 | 0.0021169 |
| Poisson’s ratio | 9 | Squared Exponential | 0.0215959 | 0.0021870 |
| Poisson’s ratio | 10 | Exponential | 0.0230327 | 0.0016475 |

The lowest mean validation NRMSE selects ARD Matern 3/2 for peak stress (0.02213) and Poisson’s ratio (0.02041), and ARD Matern 5/2 for peak strain (0.01540) and elastic modulus (0.02748). These four choices define the production model bundle.

Table S7. Nested independent-test performance and split-wise kernel-selection counts (mean ± SD, n = 5)

| Response | Production kernel | Split-wise validation selections | n | Test RMSE | Test NRMSE | Test MARE | Test R² |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Peak stress | ARD Matern 3/2 | ARD Matern 3/2 (3); ARD Exponential (1); ARD Matern 5/2 (1) | 5 | 1.55671 ± 0.07720 | 0.02433 ± 0.00091 | 0.02760 ± 0.00049 | 0.98788 ± 0.00071 |
| Peak strain | ARD Matern 5/2 | ARD Matern 5/2 (4); ARD Matern 3/2 (1) | 5 | 0.08138 ± 0.00262 | 0.01923 ± 0.00055 | 0.04162 ± 0.00194 | 0.98862 ± 0.00057 |
| Elastic modulus | ARD Matern 5/2 | ARD Matern 3/2 (1); ARD Matern 5/2 (1); ARD Rational Quadratic (1); Matern 3/2 (1); Rational Quadratic (1) | 5 | 1.43096 ± 0.04788 | 0.03139 ± 0.00119 | 0.01651 ± 0.00075 | 0.98687 ± 0.00093 |
| Poisson’s ratio | ARD Matern 3/2 | ARD Matern 3/2 (4); ARD Matern 5/2 (1) | 5 | 0.00669 ± 0.00018 | 0.01991 ± 0.00081 | 0.01443 ± 0.00086 | 0.99344 ± 0.00051 |

The production kernel minimizes mean validation of NRMSE across all five splits. The nested estimate repeats the selection inside each split, so its test metric evaluates the selection procedure rather than a single fixed kernel. The elastic-modulus selections vary across all five candidate winners, which is reported as model-selection uncertainty rather than suppressed.

S7. Surrogate-conditioned dispersion at common within-target accuracy

Supplementary Fig. S1 presents the objective-space trade-offs and parameter dispersion. The 120 final NSGA-III populations contain 7107 candidate records. For each target, a common accuracy limit was set to the lowest mean relative error across its ten final populations plus 0.005, an additive tolerance of 0.5 percentage points. This retained 2215 records. Removing exact duplicates of the five-parameter vector within each target left 2198 distinct vectors; their contributing runs are retained in the data archive. For each target, the parameter interquartile range was divided by the corresponding Table 1 domain width. The reported means give equal weight to the 12 targets. Mean normalized IQR was 0.167343 for tensile strength, 0.098602 for friction angle, 0.090315 for cohesion, 0.023744 for stiffness ratio and 0.014033 for bond effective modulus. The same ordering of target-averaged dispersion was obtained with tolerances of 0.0025 and 0.0100, although the magnitudes changed. This does not imply an identical ordering for every target. The maximum retained componentwise surrogate errors were 3.787% for peak stress, 4.850% for peak strain, 4.325% for elastic modulus and 2.194% for Poisson’s ratio. These are descriptive results of the mean-error filter, not a pre-imposed componentwise acceptance criterion. The spread is interpreted as surrogate-conditioned search dispersion, not physical parameter uncertainty.

Table S8. Target-level normalized spread among 2198 distinct retained vectors (delta = 0.005)

| Rank | Parameter | Mean normalized IQR | Median normalized IQR | Mean normalized range |
| --- | --- | --- | --- | --- |
| 1 | Tensile strength | 0.167343 | 0.141028 | 0.437601 |
| 2 | Friction angle | 0.098602 | 0.081613 | 0.514021 |
| 3 | Cohesion | 0.090315 | 0.039533 | 0.396763 |
| 4 | Stiffness ratio | 0.023744 | 0.018000 | 0.066820 |
| 5 | Bond effective modulus | 0.014033 | 0.007971 | 0.045179 |

Quartiles use linear interpolation. IQR and range are calculated within each target after removing exact duplicate five-parameter vectors. Normalization uses the Table 1 widths [60 GPa, 2, 80 MPa, 20 MPa, 60°]. The mean and median summarize the 12 target-level values with equal target weights. Exact duplicate removal does not merge numerically close but distinct vectors.

analysis_results contains retained_unique_candidates_delta005.csv, all retained occurrences, source run information, target-level counts, per-target component errors, run-best errors and sensitivity results. The older modeling_results/nonuniqueness directory is retained only as superseded provenance.

S8. Forward checks and representative NSGA-III candidates

The prescribed forward cases retain samples 1–6 and 8 after screening against the five design bounds. The twelve numerical targets have archived generating parameter–response records. All twelve current optimisation representatives are included in the new direct checks. Target generation records, surrogate predictions and direct outcomes are kept in separate tables and CSV files.

Table S9. Refitted-GPR forward-check errors for seven retained in-domain cases

| Response | n | MSE | RMSE | MARE | R² |
| --- | --- | --- | --- | --- | --- |
| Peak stress | 7 | 4.487991 | 2.118488 | 0.03813334 | 0.9721701 |
| Peak strain | 7 | 0.0039898 | 0.06316486 | 0.0322146 | 0.9911594 |
| Elastic modulus | 7 | 6.024161 | 2.454417 | 0.03333457 | 0.9494894 |
| Poisson’s ratio | 7 | 4.629887e-05 | 0.006804327 | 0.01853698 | 0.9896692 |

Table S10. Representative NSGA-III candidates and production-GPR predictions (displayed precision; full precision in CSV)

| Target | Run | Seed | Ē*c / GPa | k̄* | σ̄c / MPa | c̄ / MPa | φ̄ / ° | Pred. stress | Pred. strain | Pred. modulus | Pred. ν | Mean error |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4 | 20270919 | 23.3294 | 2.1535 | 62.8352 | 34.2678 | 38.5464 | 70.0608 | 2.8200 | 26.9892 | 0.30975 | 0.000521 |
| 2 | 7 | 20280922 | 52.0241 | 2.8637 | 36.3560 | 19.9173 | 62.3849 | 31.9505 | 0.7801 | 44.0871 | 0.40127 | 0.001694 |
| 3 | 7 | 20290922 | 73.4181 | 1.4176 | 50.7536 | 18.5094 | 55.4156 | 34.9846 | 0.6505 | 55.9843 | 0.24997 | 0.000399 |
| 4 | 7 | 20300922 | 37.7804 | 1.9674 | 19.4887 | 29.6249 | 47.5033 | 34.9724 | 0.9898 | 37.9214 | 0.31971 | 0.000985 |
| 5 | 7 | 20310922 | 42.0560 | 2.8825 | 61.9548 | 27.9273 | 64.9947 | 45.0005 | 1.2899 | 38.9970 | 0.39014 | 0.000124 |
| 6 | 10 | 20320925 | 71.0115 | 1.7803 | 43.1211 | 19.7218 | 22.1592 | 38.9351 | 0.8266 | 52.0218 | 0.30953 | 0.009391 |
| 7 | 10 | 20330925 | 39.9411 | 1.8096 | 69.7415 | 24.3925 | 29.4389 | 51.0796 | 1.4673 | 40.0377 | 0.28999 | 0.000723 |
| 8 | 1 | 20340916 | 24.0034 | 2.0821 | 68.5931 | 25.8480 | 26.3336 | 54.3693 | 2.2453 | 27.9992 | 0.31000 | 0.000121 |
| 9 | 8 | 20350923 | 28.2808 | 2.9203 | 34.0612 | 28.1023 | 56.3930 | 44.9810 | 1.6151 | 29.4941 | 0.38136 | 0.006053 |
| 10 | 8 | 20360923 | 40.6824 | 1.0011 | 15.5247 | 26.3899 | 53.9709 | 32.9034 | 0.7797 | 42.9926 | 0.17951 | 0.001537 |
| 11 | 4 | 20370919 | 18.3261 | 1.6919 | 58.8741 | 19.7477 | 29.1474 | 41.9367 | 1.9917 | 23.9683 | 0.28061 | 0.002289 |
| 12 | 3 | 20380918 | 17.6008 | 1.3069 | 24.2688 | 18.1998 | 63.9929 | 27.9498 | 1.2282 | 24.1758 | 0.24084 | 0.004846 |

Table S10 preserves the production-GPR predictions and full-precision selected inputs. The optimiser seed is not a packing seed. The same vectors are used in all twelve new direct checks; paired target, GPR and PFC results are supplied in Table S16.

S9. Reproduction of Fig. 5 grey relational degrees

All five mesoscopic and four macroscopic series were independently min–max normalized to [0,1]. For each mesoscopic–macroscopic pair, absolute pointwise differences were calculated; the pair-specific minimum and maximum differences were used in the grey relational coefficient with distinguishing coefficient ζ = 0.5. The grey relational degree is the arithmetic mean of 3125 pointwise coefficients. The supplied script reproduces the displayed Fig. 5 matrix with maximum absolute difference 0.

Table S11. Reproduced grey relational degree matrix

| Mesoscopic parameter | Peak stress | Peak strain | Elastic modulus | Poisson’s ratio |
| --- | --- | --- | --- | --- |
| Bond effective modulus | 0.632535 | 0.566652 | 0.706386 | 0.634351 |
| Stiffness ratio | 0.637174 | 0.624580 | 0.612252 | 0.655441 |
| Tensile strength | 0.677210 | 0.644445 | 0.629538 | 0.621780 |
| Cohesion | 0.701074 | 0.636192 | 0.623385 | 0.631826 |
| Friction angle | 0.610888 | 0.606404 | 0.625626 | 0.634689 |

The only value below 0.6 is bond effective modulus–peak strain (0.566652). The maximum is bond effective modulus–elastic modulus (0.706386); no displayed value exceeds 0.8.

S10. Data versions and recovered simulation records

The surrogate_analysis directory contains the datasets, saved models and numerical results for the previously reported surrogate and optimiser analyses, including run-level records for Tables S1–S3. VERSION_MANIFEST_CURRENT.csv links its dataset, models, target records and outputs to the new direct_validation directory. Table 9 now uses the current representatives. Earlier rounded calibration records and Table S15 calculations remain historical records, not replacement outcomes for current parameters.

Table S12. Sensitivity of target-averaged normalized IQR to the common target-best tolerance

| delta | Distinct vectors | Tensile strength | Friction angle | Cohesion | Stiffness ratio | Bond modulus |
| --- | --- | --- | --- | --- | --- | --- |
| 0.0025 | 837 | 0.135388 | 0.123056 | 0.104498 | 0.018102 | 0.013254 |
| 0.0050 | 2198 | 0.167343 | 0.098602 | 0.090315 | 0.023744 | 0.014033 |
| 0.0100 | 4013 | 0.186371 | 0.149171 | 0.099559 | 0.027897 | 0.015082 |

Table S13. Candidate counts under the common target-best + 0.005 criterion

| Target | Mean-error limit | Records | Distinct vectors | Contributing runs |
| --- | --- | --- | --- | --- |
| 1 | 0.005521 | 345 | 345 | 10 |
| 2 | 0.006694 | 129 | 129 | 5 |
| 3 | 0.005399 | 425 | 421 | 10 |
| 4 | 0.005985 | 252 | 251 | 9 |
| 5 | 0.005124 | 241 | 236 | 8 |
| 6 | 0.014391 | 198 | 198 | 10 |
| 7 | 0.005723 | 146 | 146 | 7 |
| 8 | 0.005121 | 130 | 128 | 4 |
| 9 | 0.011053 | 54 | 54 | 4 |
| 10 | 0.006537 | 112 | 107 | 7 |
| 11 | 0.007289 | 141 | 141 | 6 |
| 12 | 0.009846 | 42 | 42 | 5 |

Table S14. Componentwise surrogate relative errors among the 2198 distinct retained vectors

| Response | Median | 95th percentile | Maximum |
| --- | --- | --- | --- |
| Peak stress | 0.003025 | 0.016282 | 0.037866 |
| Peak strain | 0.003822 | 0.020309 | 0.048503 |
| Elastic modulus | 0.002068 | 0.013994 | 0.043252 |
| Poisson’s ratio | 0.002107 | 0.011145 | 0.021938 |

Errors in Table S14 are dimensionless fractions pooled over retained distinct vectors, not target-equally-weighted means. Per-target summaries are supplied separately. They are GPR-to-target errors, not PFC-to-target errors.

The earlier calibrated inputs and PFC2D responses are retained in final_tables/table9_historical_direct_pfc.csv at their original displayed precision. Original MARE values remain unchanged. Eight archived terminal states have parameter-matched response histories identical to the historical replay exports; this demonstrates reproduction of those histories, not identification of the original modulus and Poisson-ratio sampling positions.

An archived workbook records five parameters and four responses in Sheet1 C7:K18. Each of the twelve current targets has one matching response row at absolute tolerance 1e-8. target_workbook_provenance.csv gives the target-to-row mapping, recorded parameters and source cells. The source workbook is retained unchanged in target_source_records; Sheet2 contains separate old inverse-calibration results. The target-1 stress–strain file has a peak of 70 MPa at strain 0.00282, consistent with its recorded target values. This historical curve is not a direct check of the current Table S10 candidate.

The target workbook mapping remains in surrogate_analysis/target_source_records/target_workbook_provenance.csv. Its generating vectors are distinct from the earlier calibrated vectors. The recovered specimen-generation script specifies seed 10003, porosity 0.08, particle radii 0.35–0.581 mm and density 2650 kg/m³. The archived pre-bonding specimen has 6635 particles. New direct checks restore this identical save independently for every case. The recovered script is not proof that every historical dataset row used an independently generated packing.

For the direct checks, each parameter vector was assigned to an independent restoration of the recovered pre-bonding specimen, which contains 6635 particles. The specimen was not regenerated between cases. The original bonding and uniaxial-loading scripts were used with PFC2D 5.00 Release 025. The two platens moved at equal and opposite velocities of 0.01 m/s. The local damping coefficient was set to 0.3 during the UCS loading stage of these direct checks. Integrated measurement strains and the peak-stress accumulator were reset before loading. Loading stopped after the stress fell below 10% of its peak. Axial stress was obtained from the mean opposing platen contact force divided by specimen width, and axial strain from the change in platen separation divided by initial height. Lateral and local axial strains were obtained by integrating the strain-rate components in a central measurement circle of radius one-half the specimen width. The peak response is the maximum recorded stress and its paired axial strain. For the new direct checks, the secant modulus and Poisson ratio are read at the record nearest 50% of peak stress on the ascending branch. The secant modulus is the absolute stress-to-wall-strain ratio; the Poisson ratio is the absolute ratio of the integrated lateral and local axial strains. Neither quantity is obtained by linear regression. Supplementary Section S11 also reports readouts at 25%, 40% and 60% of peak stress. The historical dataset is retained unchanged; its original modulus and Poisson-ratio sampling positions cannot be confirmed from the recovered records.

Direct-validation claims now concern the twelve current full-precision representatives under the documented new protocol. Original GPR predictions for the different historical rounded calibration vectors are not reconstructed. The historical and current parameter sets remain separate.

Table S15. Approximate PFC-to-target errors from the displayed historical calibration records

| Case | Stress error | Strain error | Modulus error | Poisson ratio error | Approx. mean | Approx. maximum | Original reported MARE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.0286 | 0.0248 | 0.0000 | 0.0000 | 0.0133 | 0.0286 | 0.0133 |
| 2 | 0.0625 | 0.0128 | 0.0000 | 0.0250 | 0.0251 | 0.0625 | 0.0251 |
| 3 | 0.0286 | 0.0308 | 0.0179 | 0.0000 | 0.0193 | 0.0308 | 0.0193 |
| 4 | 0.0571 | 0.0505 | 0.0000 | 0.0000 | 0.0269 | 0.0571 | 0.0269 |
| 5 | 0.0444 | 0.0620 | 0.0000 | 0.0256 | 0.0330 | 0.0620 | 0.0330 |
| 6 | 0.0000 | 0.0119 | 0.0189 | 0.0645 | 0.0238 | 0.0645 | 0.0238 |
| 7 | 0.0000 | 0.0136 | 0.0000 | 0.0345 | 0.0120 | 0.0345 | 0.0126 |
| 8 | 0.0370 | 0.0222 | 0.0000 | 0.0000 | 0.0148 | 0.0370 | 0.0137 |
| 9 | 0.0889 | 0.1118 | 0.0345 | 0.0263 | 0.0654 | 0.1118 | 0.0654 |
| 10 | 0.0606 | 0.0641 | 0.0000 | 0.0000 | 0.0312 | 0.0641 | 0.0312 |
| 11 | 0.0714 | 0.0850 | 0.0417 | 0.0000 | 0.0495 | 0.0850 | 0.0495 |
| 12 | 0.0357 | 0.0492 | 0.0000 | 0.0000 | 0.0212 | 0.0492 | 0.0212 |

All errors are dimensionless absolute relative differences, |PFC − target| / |target|. The first six error columns are recomputed from rounded displayed responses, not native full-precision results. Their approximate mean must not replace the separately preserved original MARE. Differences between these two columns cannot be used to infer undocumented precision or reconstruct the original calculation. No GPR-prediction column is inferred from the PFC values.

S11. Direct checks of current representatives

All twelve representatives are included, so no case is selected or excluded according to its direct simulation outcome. The mean PFC-to-target MARE was 3.10%, with a range of 0.62–5.81%. The largest componentwise error was 14.28%. Componentwise errors, the unweighted mean error and the maximum error are reported for each case. The modulus and Poisson-ratio comparison uses the explicit new readout in Section 2.1; the original sampling positions of the archived target responses are not established. The direct checks therefore quantify agreement with the fixed archived targets under this documented protocol, rather than proving that every response matches within 5%. Historical calibration records are retained separately in the data archive and are not substituted for the current vectors.

Table S16. Paired target, GPR and direct PFC2D responses and componentwise relative errors

| Case | Response | Target | GPR | PFC | PFC error (%) | GPR error (%) | PFC–GPR gap (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Peak stress (MPa) | 70 | 70.0608 | 74.9181 | 7.026 | 0.087 | 6.939 |
| 1 | Peak strain (‰) | 2.82 | 2.82004 | 2.94465 | 4.420 | 0.001 | 4.419 |
| 1 | Secant modulus (GPa) | 27 | 26.9892 | 26.99 | 0.037 | 0.040 | 0.003 |
| 1 | Poisson ratio | 0.31 | 0.309751 | 0.314573 | 1.475 | 0.080 | 1.555 |
| 2 | Peak stress (MPa) | 32 | 31.9505 | 31.1049 | 2.797 | 0.155 | 2.642 |
| 2 | Peak strain (‰) | 0.78 | 0.780056 | 0.764878 | 1.939 | 0.007 | 1.946 |
| 2 | Secant modulus (GPa) | 44 | 44.0871 | 44.4946 | 1.124 | 0.198 | 0.926 |
| 2 | Poisson ratio | 0.4 | 0.401271 | 0.415761 | 3.940 | 0.318 | 3.622 |
| 3 | Peak stress (MPa) | 35 | 34.9846 | 35.8858 | 2.531 | 0.044 | 2.575 |
| 3 | Peak strain (‰) | 0.65 | 0.650479 | 0.667354 | 2.670 | 0.074 | 2.596 |
| 3 | Secant modulus (GPa) | 56 | 55.9843 | 57.4407 | 2.573 | 0.028 | 2.601 |
| 3 | Poisson ratio | 0.25 | 0.249965 | 0.267478 | 6.991 | 0.014 | 7.005 |
| 4 | Peak stress (MPa) | 35 | 34.9724 | 38.7574 | 10.736 | 0.079 | 10.814 |
| 4 | Peak strain (‰) | 0.99 | 0.989812 | 1.07895 | 8.985 | 0.019 | 9.004 |
| 4 | Secant modulus (GPa) | 38 | 37.9214 | 38.124 | 0.326 | 0.207 | 0.533 |
| 4 | Poisson ratio | 0.32 | 0.319714 | 0.330162 | 3.176 | 0.089 | 3.265 |
| 5 | Peak stress (MPa) | 45 | 45.0005 | 45.3976 | 0.884 | 0.001 | 0.883 |
| 5 | Peak strain (‰) | 1.29 | 1.28995 | 1.29353 | 0.273 | 0.004 | 0.277 |
| 5 | Secant modulus (GPa) | 39 | 38.997 | 38.6736 | 0.837 | 0.008 | 0.829 |
| 5 | Poisson ratio | 0.39 | 0.390144 | 0.391888 | 0.484 | 0.037 | 0.447 |
| 6 | Peak stress (MPa) | 39 | 38.9351 | 39.3165 | 0.811 | 0.166 | 0.978 |
| 6 | Peak strain (‰) | 0.84 | 0.826622 | 0.875726 | 4.253 | 1.593 | 5.846 |
| 6 | Secant modulus (GPa) | 53 | 52.0218 | 55.0065 | 3.786 | 1.846 | 5.632 |
| 6 | Poisson ratio | 0.31 | 0.309529 | 0.306699 | 1.065 | 0.152 | 0.913 |
| 7 | Peak stress (MPa) | 51 | 51.0796 | 50.5083 | 0.964 | 0.156 | 1.120 |
| 7 | Peak strain (‰) | 1.46679 | 1.46732 | 1.45169 | 1.029 | 0.036 | 1.065 |
| 7 | Secant modulus (GPa) | 40 | 40.0377 | 40.1859 | 0.465 | 0.094 | 0.370 |
| 7 | Poisson ratio | 0.29 | 0.289993 | 0.295266 | 1.816 | 0.002 | 1.818 |
| 8 | Peak stress (MPa) | 54.3502 | 54.3693 | 54.5593 | 0.385 | 0.035 | 0.350 |
| 8 | Peak strain (‰) | 2.24513 | 2.24533 | 2.21472 | 1.354 | 0.009 | 1.363 |
| 8 | Secant modulus (GPa) | 28 | 27.9992 | 27.9694 | 0.109 | 0.003 | 0.106 |
| 8 | Poisson ratio | 0.31 | 0.309995 | 0.31946 | 3.052 | 0.002 | 3.053 |
| 9 | Peak stress (MPa) | 45 | 44.981 | 45.0198 | 0.044 | 0.042 | 0.086 |
| 9 | Peak strain (‰) | 1.61 | 1.61509 | 1.583 | 1.677 | 0.316 | 1.993 |
| 9 | Secant modulus (GPa) | 29 | 29.4941 | 29.6596 | 2.274 | 1.704 | 0.571 |
| 9 | Poisson ratio | 0.38 | 0.381363 | 0.394994 | 3.946 | 0.359 | 3.587 |
| 10 | Peak stress (MPa) | 33 | 32.9034 | 33.7342 | 2.225 | 0.293 | 2.518 |
| 10 | Peak strain (‰) | 0.78 | 0.779742 | 0.80717 | 3.483 | 0.033 | 3.516 |
| 10 | Secant modulus (GPa) | 43 | 42.9926 | 44.1529 | 2.681 | 0.017 | 2.698 |
| 10 | Poisson ratio | 0.18 | 0.17951 | 0.205711 | 14.284 | 0.272 | 14.556 |
| 11 | Peak stress (MPa) | 42 | 41.9367 | 45.0688 | 7.307 | 0.151 | 7.457 |
| 11 | Peak strain (‰) | 2 | 1.99172 | 2.14078 | 7.039 | 0.414 | 7.453 |
| 11 | Secant modulus (GPa) | 24 | 23.9683 | 23.9471 | 0.220 | 0.132 | 0.088 |
| 11 | Poisson ratio | 0.28 | 0.280613 | 0.287248 | 2.588 | 0.219 | 2.370 |
| 12 | Peak stress (MPa) | 28 | 27.9498 | 28.0462 | 0.165 | 0.179 | 0.344 |
| 12 | Peak strain (‰) | 1.22 | 1.22824 | 1.19666 | 1.913 | 0.675 | 2.588 |
| 12 | Secant modulus (GPa) | 24 | 24.1758 | 24.9132 | 3.805 | 0.733 | 3.072 |
| 12 | Poisson ratio | 0.24 | 0.240843 | 0.271183 | 12.993 | 0.351 | 12.642 |

Errors are absolute differences divided by the absolute target. The PFC–GPR gap is also divided by the target. Target modulus and Poisson ratio retain their archived meanings; the new direct readout follows Section 2.1. These quantities are not assumed to share an undocumented historical sampling rule.

Table S17. Direct-check mean and maximum response errors

| Case | MARE (%) | Maximum error (%) |
| --- | --- | --- |
| 1 | 3.239 | 7.026 |
| 2 | 2.450 | 3.940 |
| 3 | 3.691 | 6.991 |
| 4 | 5.806 | 10.736 |
| 5 | 0.619 | 0.884 |
| 6 | 2.479 | 4.253 |
| 7 | 1.068 | 1.816 |
| 8 | 1.225 | 3.052 |
| 9 | 1.985 | 3.946 |
| 10 | 5.668 | 14.284 |
| 11 | 4.289 | 7.307 |
| 12 | 4.719 | 12.993 |

Table S18. Secant modulus (GPa) at fixed ascending-branch stress fractions

| Case | 25% | 40% | 50% (primary) | 60% |
| --- | --- | --- | --- | --- |
| 1 | 27.571 | 27.1627 | 26.99 | 26.9029 |
| 2 | 46.7797 | 45.0138 | 44.4946 | 44.1462 |
| 3 | 60.0104 | 58.0385 | 57.4407 | 57.0523 |
| 4 | 40.2323 | 38.6251 | 38.124 | 37.8719 |
| 5 | 39.9 | 38.9747 | 38.6736 | 38.4057 |
| 6 | 57.1131 | 55.5045 | 55.0065 | 54.4784 |
| 7 | 41.3835 | 40.4793 | 40.1859 | 39.9829 |
| 8 | 28.751 | 28.1614 | 27.9694 | 27.8345 |
| 9 | 30.6683 | 29.945 | 29.6596 | 29.4899 |
| 10 | 47.1677 | 44.7764 | 44.1529 | 43.7702 |
| 11 | 24.8056 | 24.1566 | 23.9471 | 23.8008 |
| 12 | 26.5607 | 25.2949 | 24.9132 | 24.6702 |

Table S19. Poisson ratio at fixed ascending-branch stress fractions

| Case | 25% | 40% | 50% (primary) | 60% |
| --- | --- | --- | --- | --- |
| 1 | 0.346952 | 0.322136 | 0.314573 | 0.309293 |
| 2 | 0.500875 | 0.436052 | 0.415761 | 0.402549 |
| 3 | 0.34717 | 0.286452 | 0.267478 | 0.254885 |
| 4 | 0.396864 | 0.348755 | 0.330162 | 0.318661 |
| 5 | 0.447113 | 0.405255 | 0.391888 | 0.38579 |
| 6 | 0.376469 | 0.323478 | 0.306699 | 0.296571 |
| 7 | 0.347287 | 0.307887 | 0.295266 | 0.287067 |
| 8 | 0.366121 | 0.330817 | 0.31946 | 0.312178 |
| 9 | 0.450045 | 0.407931 | 0.394994 | 0.386293 |
| 10 | 0.292331 | 0.228145 | 0.205711 | 0.191816 |
| 11 | 0.346349 | 0.301503 | 0.287248 | 0.277977 |
| 12 | 0.378498 | 0.296379 | 0.271183 | 0.254965 |

For each fraction, the earliest sampled record minimising the absolute difference from the specified peak-stress fraction is used. The protocol was fixed before inspecting this batch. No fraction is selected afterwards to minimise the response error.